# Shokz OpenComm2 2025 Accessible User Guide

Contents

How It Works	2

Headset Controls	2

Turn on the headset:	2

Turn off the headset:	2

Pairing to Mobile Device	2

How to Wear	2

Music and Calls	2

How to check battery status:	4

EQ Modes	4

Switching EQ Modes	4

Turn MultiPoint Pairing On	4

Turn MultiPoint Pairing Off	5

Restore Factory Settings	5

Changing Languages	5

Charging Tips	6

Warranty	6

Troubleshooting	6

What’s in the Box	6

Shokz App & Shokz Connect	7

# How It Works

## Headset Controls

### Turn on the headset:

Press and hold the Power/Volume+ Button until the LED indicator flashes blue.

*Audrey will say, “Welcome to Shokz.”

### Turn off the headset:

Press and hold the Power/Volume+ Button until the LED indicator flashes red.

*Audrey will say, “Power off.”

## Pairing to Mobile Device

Start with your headset turned off.

Press and hold the Volume+ Button until the LED indicator flashes red and blue alternately.

Select "OpenComm2 by Shokz_II" on your device's Bluetooth settings. Audrey will say, "Connected".

## How to Wear

Slide the headset over your head, bring it to the back of your neck and position the ear hooks on your ears.

*Position the noise-canceling microphone close to your mouth.

*Wearing your headset the wrong way could cause discomfort. Be sure you have the transducers resting in front of your ears.

## Music and Calls

Adjust Volume:

- Press the Volume+ or Volume– button during playback to adjust volume.

- A “beep” will sound when you reach maximum volume.

Play/Pause Music:

- Press the Multifunction button once.

Next Track:

- Double-press the Multifunction button.

Previous Track:

- Triple-press the Multifunction button.

Answer a Call:

- Press once when a call comes in.

End a Call:

- Press once during a call.

Reject a Call:

- Double-press the Multifunction button when a call comes in.

Answer a Second Call and End the First:

- Press once during the new incoming call.

Put Current Call on Hold and Answer New Call:

- Press and hold the Multifunction button for 2 seconds.

Switch Between Two Calls:

- Press and hold the Multifunction button for 2 seconds.

End a Call and Switch to the Call on Hold:

- Press once.

Mute Microphone:

- Press the Mute button once while on a call.
Audrey will say “Mute on.”

Unmute Microphone:

- Press the Mute button again.
Audrey will say “Mute off.”

Activate Voice Assistant (e.g., Siri or Google Assistant):

- Press and hold the Mute button for 2 seconds when not on a call.

### How to check battery status:

Click the “Volume+” or “Volume-” button while music is paused, Audrey will tell you what the battery status is:

* "Battery high/Battery medium/Battery low/Charge me"

## EQ Modes

Standard Mode: Balanced frequency for listening to music

Vocal Booster Mode: Enhanced intermediate frequencies for audiobooks, podcasts, etc.

## Switching EQ Modes

Press and hold both buttons (Power/Volume+ Button and Volume- Button) while music is playing until you hear a beep.

*Audrey will say, “Standard Mode/Vocal Booster Mode.”

## Turn MultiPoint Pairing On

Start with your headset powered off.

Press and hold Volume+ until Audrey will say “Pairing” and the LED indicator flashes red and blue alternately.

Press and hold the multifunction button and Volume+ button until Audrey will say "MultiPoint enabled."

Open first device's Bluetooth® menu and select "OpenComm2 by Shokz_II"Audrey will say "Connected".

Turn your headset off.

Re-enter pairing mode by pressing and holding Volume+ until Audrey says “Pairing”and the LED indicator flashes red and blue alternately.

Open second device's Bluetooth® menu and select "OpenComm2 by Shokz_II"Audrey will say "Connected."

Turn your headset off.

Turn your headset on. Your headset is now connected to both devices.

## Turn MultiPoint Pairing Off

Start with your headset powered off.

Press and hold Volume+ until Audrey will say “Pairing” and the LED indicator flashes red and blue alternately.

Press and hold the multifunction button and Volume+ button until Audrey will say "MultiPoint Pairing disabled."

## Restore Factory Settings

Start with your headset powered off.

Press and hold the “Volume+” button until Audrey says “Pairing”. The LED indicator will flash red and blue.

Hold down the Multifunction button, the Volume- button, and the Volume+ button until you hear two beeps.

Turn your headset off. The headset is now reset and can be re-paired to your device.

## Changing Languages

Your headset has four built-in languages:

Chinese, English, Japanese and Korean.

Start with your headset powered off.

Press and hold Volume+ until Audrey says “Pairing” and the LED indicator flashes red and blue alternately.

Double-press the multifunction button. Languages have been switched when you hear the corresponding language.

## Charging Tips

The headset will automatically shut down when charging

When the headset's battery is low, you will hear a “Charge me” prompt, and the red light will flash. Use the provided charging cable to recharge.

The LED indicator will light up red while charging, and the headset will automatically power off. Once fully charged, the LED indicator will turn from red to blue.

## Warranty

Your headset is covered by a warranty. Visit our website at https://shokz.com/pages/warranty-landing for details of the limited warranty. *Failure to register will not affect your limited warranty rights.

# Troubleshooting

If you experience any issues or problems, please visit https://shokz.com/pages/faq for more support

# What’s in the Box

OpenComm2 Headset

USB-C Charging Cable

OpenComm2 User Guide

Legal Statement

*Please note that product packaging may vary depending on the sales region.

If any part of your product is damaged or missing, do not use it. Contact an authorized Shokz dealer or Shokz customer service.

# Shokz App & Shokz Connect

Download the Shokz App for your phone and Shokz Connect for your computer for the latest features and technical support.

Please note, the headset only supports the Android version of the Shokz App.

Google Play is a trademark of Google LLC.

"Note: The wireless adapter (Loop120 UC) mentioned in other guides is not included with this version. For more information about adapter pairing, visit the Shokz website."