# TRIBIT STORMBOX LAVA PORTABLE WIRELESS SPEAKER

Accessible User Guide (English)

TRIBIT StormBox Lava Portable Wireless Speaker
Model: BTS51

## What’s in the Box

- StormBox Lava Portable Wireless Speaker

- USB-C Charging Cable

- USB-A Output Port

- Carrying Straps

- User Documentation

## Button Press Definitions

These symbols describe how to press any referenced button:

- x1 = Press once

- x2 = Press twice

- x3 = Press three times

- 2S = Press and hold for 2 seconds

- Reset = Hold the + and – buttons together for 5 seconds

## Charging the Speaker

- When the battery is below 20 percent, a red light blinks.

- While charging, the red light stays on.

- When fully charged (about 5 hours), the red light turns off.

Note: Keep the charging port and cable completely dry while charging.

## Charging Other Devices

You can charge external devices using the speaker’s USB-A port. The speaker will stop charging other devices if its own battery drops below 20 percent.

## Power On and Off

- Press the Power button (x1) to turn the speaker on or off.

- If there is no audio or any button activity, the speaker will automatically turn off after 75 minutes.

- To force a shutdown, press and hold the Power button (2S), then restart the speaker.

## Pairing (Bluetooth)

- Press the Bluetooth button (x1) to enter pairing mode.

- Press and hold the Bluetooth button (2S) to disconnect the Bluetooth connection.

- The speaker can connect to up to two Bluetooth devices at the same time.

Note: If Bluetooth and a wired AUX connection are both active, press the Bluetooth button (x2) to switch between them.

## Button Layout (Left to Right)

- Power Button

- Bluetooth Button

- Minus Button (Volume Down)

- Multifunction Circle Button (Play / Pause)

- Plus Button (Volume Up)

- XBass Button

- TWS Button (Stereo Mode and Party Mode)

## Stereo Mode (TWS)

You can pair two StormBox Lava speakers into a Left and Right stereo pair.

- Connect Speaker A to your phone’s Bluetooth.

- Make sure Speaker B is not connected to any device.

- Press the TWS button (x1) on both speakers.

- Once paired:

  - Speaker A becomes the left channel.

  - Speaker B becomes the right channel.

- To exit Stereo Mode, press and hold the TWS button (2S) on either speaker.

## Party Mode (Up to 100+ Speakers)

You can link multiple StormBox Lava speakers for synchronized playback.

- Connect the host speaker A to your phone’s Bluetooth.

- Press the TWS button (x2) on the host speaker.

- Make sure all other speakers are not connected to any device.

- Press the TWS button (x2) on each additional speaker.

- Each speaker will announce when it successfully joins the group.

- Up to 100+ speakers can play together.

- To exit Party Mode, press and hold the TWS button (2S) on any speaker.

## Tribit App

Use the Tribit app to:

- Adjust EQ settings

- Update firmware

- Access additional features

## Safety Precautions

- Ensure the silicone plug on the charging port is tightly closed before exposure to water.

- Keep the charging port and cable completely dry during charging.

- Do not drop the speaker or place it where it can fall.

- Do not disassemble, modify, or attempt to repair the speaker.

- This product contains a lithium battery. Do not burn or expose it to excessive heat.

- Use a power adapter that meets local safety standards.

- Warranty period: 12 months.

## Specifications

- Bluetooth Version: V5.4

- Output Power: 80W total (30W × 2 woofers, 10W × 2 tweeters)

- Bluetooth Protocols: A2DP, AVRCP, HFP, HSP

- Frequency Response: 43Hz to 20kHz

- Frequency Band: 2402 to 2480MHz

- Maximum Transmission Power: ≤ 6 dBm

- Battery Capacity: 43.2Wh

- Play Time: Up to 24 hours (50 percent volume, XBass Off)

- Charging Time: About 5 hours with a 15W charger

- Waterproof Rating: IP67

- Dimensions: 310mm × 147mm × 152mm

- Dimensions (inches): 12.2 × 5.8 × 6.0

- Weight: 2300g (5.07 lbs)

## FCC Statement

This device complies with Part 15 of the FCC Rules. Operation is subject to:

- This device may not cause harmful interference.

- This device must accept any interference received, including interference that may cause undesired operation.

If interference occurs:

- Reorient or relocate the receiving antenna.

- Increase separation between the equipment and receiver.

- Connect the device to a different circuit.

- Consult a radio or TV technician.

Unauthorized modifications may void FCC authorization.

## IC Statement (Canada)

This device complies with Industry Canada licence-exempt RSS standards. Operation is subject to:

- This device may not cause interference.

- This device must accept any interference received, including interference that may cause undesired operation.

## Support Information

- For proper charging, use a power adapter between 2W and 15W.

- Compliance documents cover multiple Tribit models, including BTS51.